|  | Simulation of Leg_Triangle Date: 04 October 2026 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 3 Mesh information 4 Study Results 6 Conclusion 8. |
| --- | --- |
| # Description Static structural analysis of one 6061-T6 aluminium hexapod leg subjected to a 30N downward load. This case represents a higher loading condition to evaluate the leg's structural margin beyond nominal static loading. |  |

# Assumptions

# Model Information Model name: Leg_Triangle Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Boss-Extrude1; Solid Body; Mass:0.00670318 kg Volume:2.48266e-06 m^3 Density:2,700 kg/m^3 Weight:0.0656912 N; C:\Users\E.Kay\ANTARES PROJECT\HEXAPOD\Leg_Triangle.SLDPRT Oct 3 17:27:41 2026.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.75e+08 N/m^2. Tensile strength:; 3.1e+08 N/m^2.; SolidBody 1(Boss-Extrude1)(Leg_Triangle).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-2; Entities:; 2 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-2 (:Per item: 30 N:); Entities:; 1 face(s). Type:; Apply normal force. Value:; 30 N..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points for High quality mesh; 16 Points. Element Size; 1.35465 mm. Tolerance; 0.0677323 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 13114. Total Elements; 7741. Maximum Aspect Ratio; 4.157. % of elements with Aspect Ratio < 3; 99.8. Percentage of elements with Aspect Ratio > 10; 0. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:10. Computer name:.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 2.988e+01N/m^2 Node: 8499; 1.194e+06N/m^2 Node: 41. Leg_Triangle-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0.000e+00mm Node: 1; 6.065e-04mm Node: 89. Leg_Triangle-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed shape. Leg_Triangle-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 2.303e+02 Node: 41; 9.204e+06 Node: 8499. Leg_Triangle-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion Under applied 30N load, the leg remains within the defined material strength limit. Maximum displacement was 6.065e-4mm and minimum factor of safety was 230.327. The results indicate that the leg maintains structural integrity under the increased static load.