|  | Simulation of Upper Valve-Chasis Date: 04 October 2026 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 3 Mesh information 4 Study Results 6 Conclusion 8. |
| --- | --- |
| # Description Evaluated under 20kPa soil overburden. |  |

# Assumptions

# Model Information Model name: Upper Valve-Chasis Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Fillet2; Solid Body; Mass:0.162512 kg Volume:0.000119494 m^3 Density:1,360 kg/m^3 Weight:1.59261 N; C:\Users\E.Kay\ANTARES PROJECT\HEXAPOD\Upper Valve-Chasis.SLDPRT Oct 4 16:23:25 2026.

# Material Properties Model Reference; Properties; Components. Name:; PA6-GF30. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 1.3e+08 N/m^2. Tensile strength:; 1.3e+08 N/m^2.; SolidBody 1(Fillet2)(Upper Valve-Chasis).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-2; Entities:; 1 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Pressure-1 (:20,000 N/m^2:); Entities:; 1 face(s). Type:; Normal to selected face. Value:; 20,000. Units:; N/m^2..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points for High quality mesh; 16 Points. Element Size; 4.92696 mm. Tolerance; 0.246348 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 15066. Total Elements; 7983. Maximum Aspect Ratio; 5.9018. % of elements with Aspect Ratio < 3; 98.8. Percentage of elements with Aspect Ratio > 10; 0. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:09. Computer name:.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 1.494e+03N/m^2 Node: 14514; 2.207e+06N/m^2 Node: 2889. Upper Valve-Chasis-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0.000e+00mm Node: 74; 4.970e-02mm Node: 12282. Upper Valve-Chasis-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed shape. Upper Valve-Chasis-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 5.891e+01 Node: 2889; 8.702e+04 Node: 14514. Upper Valve-Chasis-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion The upper chasis was evaluated under an assumed 10kPa soil-overburden pressure representing approximately 0.5m of soil cover, with an additional 20kPa sensitivity case. The chassis showed minimal deformation and maintained a large factor of safety under the tested static conditions.